# ✅ OCR Bootcamp Notes (3 Hours)

Duration: 3 Hours | Level: Beginner to Intermediate

## ✅ Session Plan

| Time | Topic |
| --- | --- |
| 0:00 – 0:15 | Introduction to OCR + Real-world Uses |
| 0:15 – 0:45 | Image Preprocessing for OCR |
| 0:45 – 1:30 | OCR with Tesseract & Python |
| 1:30 – 2:15 | Advanced OCR Concepts + Accuracy Tips |
| 2:15 – 3:00 | Mini Projects + Practice + Q&A |

## ✅ PART 1: Introduction to OCR (15 min)

### What is OCR?

- Optical Character Recognition converts text from images → editable text.

- Used in document digitization, ID verification, invoice automation.

### Real-world Applications

✔ Scanning books into eBooks
 ✔ License plate recognition
 ✔ Passport/ID verification
 ✔ Healthcare data entry automation

### OCR vs ICR

- OCR: Recognizes printed text.

- ICR: Recognizes handwritten text using AI.

## ✅ PART 2: Image Preprocessing (30 min)

### Why is it important?

Poor image quality → Low OCR accuracy.
 Preprocessing improves results.

### Steps:

- Convert to Grayscale

- Apply Thresholding (Binary / Adaptive)

- Remove Noise

- Deskew text

### Example Code:

import cv2

img = cv2.imread('document.jpg')

gray = cv2.cvtColor(img, cv2.COLOR_BGR2GRAY)

thresh = cv2.threshold(gray, 150, 255, cv2.THRESH_BINARY)[1]

cv2.imwrite('processed.jpg', thresh)

## ✅ PART 3: OCR with Tesseract & Python (45 min)

### Install:

pip install pytesseract opencv-python

Also install Tesseract Engine from https://github.com/tesseract-ocr/tesseract.

### Basic OCR Example:

import pytesseract

from PIL import Image

img = Image.open('text_image.png')

text = pytesseract.image_to_string(img)

print("Extracted Text:", text)

### OCR from Preprocessed Image:

import cv2

import pytesseract

img = cv2.imread('processed.jpg')

text = pytesseract.image_to_string(img)

print(text)

### Extract Text from PDF:

from pdf2image import convert_from_path

import pytesseract

pages = convert_from_path('document.pdf', 300)

for page in pages:

text = pytesseract.image_to_string(page)

print(text)

## ✅ PART 4: Advanced OCR + Accuracy Tips (45 min)

### Challenges:

- Noisy images

- Different fonts

- Low resolution

### Solutions:

✔ Use deskewing & denoising
 ✔ Convert to grayscale
 ✔ Apply adaptive thresholding
 ✔ Use deep learning OCR (EasyOCR)

Install EasyOCR:

pip install easyocr

Code:

import easyocr

reader = easyocr.Reader(['en'])

result = reader.readtext('document.jpg')

print(result)

## ✅ PART 5: Mini Projects (45 min)

### ✅ Project 1: Extract Text from Image

- Take a sample image (receipt, document).

- Preprocess image using OpenCV.

- Extract text using Tesseract.

### ✅ Project 2: Real-time OCR from Webcam

import cv2

import pytesseract

cap = cv2.VideoCapture(0)

while True:

ret, frame = cap.read()

text = pytesseract.image_to_string(frame)

print(text)

cv2.imshow('Live OCR', frame)

if cv2.waitKey(1) & 0xFF == 27:

break

cap.release()

cv2.destroyAllWindows()

### ✅ Project 3: OCR for PDF

Convert PDF → Images → Text (already shown above).

## ✅ Practice Questions

- What is OCR and how does it work?

- Why is preprocessing important for OCR?

- Write a Python script to extract text from an image.

- How can you improve OCR accuracy?

- Difference between OCR and ICR.

## ✅ Assignment

✔ Extract text from 3 sample scanned documents and save it in a text file.
 ✔ Create a mini app (CLI or Flask) that takes an image as input and returns extracted text.

## ✅ Tools & Libraries

- OpenCV – Image Preprocessing

- pytesseract – OCR Engine

- pdf2image – Convert PDFs

- EasyOCR – Advanced OCR

### ✅ Summary

✔ OCR converts image text to editable text.
 ✔ Preprocessing is key for accuracy.
 ✔ Tesseract + Python = Simple OCR solution.
 ✔ Use EasyOCR or PaddleOCR for advanced scenarios.